COMMON LABORATORY TECHNIQUES

Reading a Meniscus

In all volumetric glassware (pipet, buret, volumetric flasks, graduated cylinder, etc.), it is necessary to read the level of a liquid. A liquid

in a small-diameter container will form a meniscus or curve at the surface of the liquid.  Usually this meniscus curves downward to a minimum at the center. To read the level of the liquid properly, the eye should be at the same level as the bottom of the meniscus. Sometimes a white card or a white card with a black mark on it will help a person to see the meniscus clearly. For volumetric flasks and transfer pipets, the volume of the glassware is exact when the bottom of the meniscus is even with the etched line. In a graduated cylinder or a buret, the volume is read from the graduations etched on the glass.  To read the volume correctly, visualize the distance between the tenths of milliliter marks as divided into ten equal amounts.

The volume is then found by reading the number of tenths of milliliters and estimating hundredths of

milliliters.

100

90

Graduated

Cylinder

91.7

46

47

Buret

46.3

# Titration

The titration  procedure  is  accomplished  by  adding small   amounts  of  one  reactant  to a beaker  or  flask containing the  other until the reaction is completed. In an acid-base  reaction,  the   reaction  is  complete  when  the amount of acid equals the	amount  of base.   The experimenter is aware of this condition by the change in pH that  occurs. The  change  in  pH  is  indicated  by  a  color change of an indicator or by a pH meter.  In other types of reactions,  the  completion  of  the  reaction  is  usually  also indicated by  a color  change. Titrations are	often accomplished using a buret. A small amount (5 - 10 ml) of the solution is used to rinse a clean buret. The rinse solution is discarded. The buret is filled to just below the zero mark.  The liquid level is read and recorded as the initial volume. A buret has zero at the top as shown in the picture. Small amounts of the solution are allowed to flow out of the buret into the flask by turning the stopcock.  Often localized color changes will be evident for a short period of time.   Mixing the solution by swirling the flask will usually cause the color to return to the original color.  As the color lasts longer, the next addition of solution needs to be smaller. Close to the endpoint (reaction completion), the additions are one drop at a time.  When the endpoint is reached, the liquid level is read and recorded as the final volume.  The volume added is the final volume minus initial volume.

VOCABULARY

| accomplish - wykonać balance – waga beaker – zlewka buret/burette – biureta burner - palnik clay triangle – trójkąt do tygli (clay – glina) crucible – tygiel desiccator – eksykator discard – pozbyć się dispense - rozdzielać dropper – wkraplacz / zakraplacz, kroplomierz etched – wytrawiony (kwasem) filter paper - sączek flask – kolba (flat-bottom – płaskodenna; funnel – lejek (separatory funnel – rozdzielacz); przelewać lejkiem indicator - wskaźnik laboratory apparatus – aparatura laboratoryjna round-bottom – okrągłodenna; three-neck – trójszyjna; volumetric – miarowa; Erienmeyer - stożkowa) | fume hood – wyciąg (dygestorium) graduated cylinder – cylinder miarowy graduation – tu: podziałka laboratory equipment – sprzęt laboratoryjny mortar – moździerz noxious – trujący potassium permanganate – nadmanganian potasu pH-meter – pH metr pipet/pipette – pipeta, odmierzać pipetą rinse - przepłukać sample - próbka species – gatunek / -ki stirrer – mieszadło (magnetic stirrer – mieszadło magnetyczne) stopcock – zawór odcinający, korek titrate - miareczkować titration - miareczkowanie tongs – szczypce tube – probówka tube holder – łapa do probówek watch glass – szkiełko zegarkowe |  |
| --- | --- | --- |

| 1) dropper |  | A) equipment used to ventilate noxious or harmful gases |
| --- | --- | --- |
| 2) beaker |  | B) apparatus used to dispense small quantities of liquids |
| 3) pipet |  | C) reaction container usually made of glass, used to hold liquid or solid samples |
| 4) funnel |  | D) a solution of known concentration which is added (titrated) to another solution to determine the concentration of a second chemical species |
| 5) fume hood |  | E) apparatus used for addition of liquids drop by drop |
| 6) titrant |  | F) apparatus used for funneling liquids from one container to another or for filtering when equipped with filter paper |

### Zad. 1 Match the words with their definitions

Zad. 2 Translate the sentences below into English:

- Przegapiłem punkt końcowy. Znów dodałem za dużo roztworu nadmanganianu potasu.

...............................................................................................................................................................

- Gdyby Jack użył mieszadła magnetycznego zamiast łyżeczki, nie zbiłby zlewki.

..............................................................................................................................................................

- Miareczkowałem wczoraj przez 3 godziny. Ból pleców mnie zabija!

..............................................................................................................................................................

- Zamierzam przepłukać kolbę okrągłodenną i odmierzyć pipetą 2 mililitry wskaźnika.

.............................................................................................................................................................

#### Zad. 3 Choose the correct answer

a) You need to shake / swirl / swing the flask to check if the titrated solution returns to its original color.

b) If you want to read the level of liquid in burette properly, keep your eye on / above / below the level of meniscus.

c) In acid-base reaction, you achieve the endpoint when the amount of acid is bigger than / equals / is smaller than the amount of base.

d) A buret has zero at the top / at the bottom / anywhere you want.

e) In titration, as the color of analyzed solution lasts longer, but vanishes, you need to add smaller / bigger / no more portion of titrant.